Data acquisition with the FemStuc

- Do not put a high voltage on the detector if the FEC boards do not have a 5-V power supply.

- The high voltage for the detector should be carefully increased or decreased by checking the leak current.  Keep it under 20 nA.

Initial setup

- Make sure that the all the HVs for the detector are turned off.

- Connect the FEC 80-pin connector to the 44-pin connector of the TPC chamber with the 80-pin-to-44pin adopter.

- Connect the external trigger cable to the “SCA STOP” lemo connector of the FEMR board.

- Connect the 5-V DC power supply to the FEC and FEMR boards.

- Turn on the 5-V DC power supply.

- Log in the laptop PC. User; suzuki   Pass;Attpc2009

- Connect the USB cable from the STUC to the USB port of the laptop. A red light on the FEMR should light up.

- Launch the CMD_SERVER.EXE (The short cut can be found on the Desktop. ). If FEC and FEMR power is indeed on, then a cmd window is launched that contains setup info for the DCC. If the last line reads “0FEM(0) supports 1FEC()” then the setup is ready.

- Put a high voltage on the detector.

Acquisition with the T2K_DAQ (based on the Labview)  -  slow DAQ with a signal monitor

- Launch the T2K_DAQ. (shortcut on the Desktop)

- Change the configuration:

  - IP address -> 127.0.0.1 press check inside box in top menu bar to save changes

  - End canal  -> 77 (in AregCluster box).

  - DAQ Pulser -> off (light must be grey, not green)

  - Pulser on/off -> off in ‘slow control parameters’ tab, off=grey

  - Change Soft sca stop -> Hard sca stop (external trigger mode) by clicking on button labeled ‘Soft SCA Stop’

  - (If you want to save the data)

    - Save data -> on button in bottom left panel, green = on

    - Specify the file name

    - Specify the number of events in one file (The box is located at top right) Note that from the second file, the file can contain only one event par file.

- ASIC gain & peaking time can be changed in ‘slow control parameters’

- Push the right arrow button for launching the Labview (in top menu bar).

- Push Start acquisition

- (When stop the acquisition) Push Stop acquisition.

Acquisition with the DaqT2K (based on cpp) – fast DAQ w/o a signal monitor

- Launch the DaqT2K. (shortcut to DaqT2K.exe on the Desktop)

- Change “Path” ->   My Documents/deployement/DAQT2K/DCCDATA

- Select an existing configuration file.  (CFG_[run#].xml)

- Change the configurations for your measurement.

  - RUN; Max event number to be recorded, Trigger condition … etc

  - NETWORK; IP address (this should be the one that displayed in the cmd server.)… etc

  - DCC; Configurations of the ASIC

  - If you want to change the blue character variables: click the right button and select ‘override’. This procedure gives you a permission to change the value. The variable becomes red. If you want to go back to the default value, click the right button and choose ‘delete’.

  - As for the black variables, you can change it just by double clicking the value.

- Reset -> Script -> Configure -> Start Run

- (In the prompted window) Confirm the run#.