import h5py

import numpy as np

import matplotlib.pyplot as plt

from tqdm import tqdm

import scipy.optimize as opt

runs = [18,19,20,21,22,23,24,25,26,27]

timestamps_list = []

zscale = 0.65

for run in  runs:

path = "/Volumes/Extreme SSD/e24joe/"

#base_path = path + 'run_%04d_raw_data_export/'%run

base_path = path + 'run_%04d/run_%04dp10_2000torr/'%(run,run)

run = 'run_%04d.h5'%run

with h5py.File(path + run) as f:

first_event, last_event = int(f['meta']['meta'][0]), int(f['meta']['meta'][2])

first_timestamp = f['meta']['meta'][1]

counts = np.load(base_path+'counts.npy')

dxy = np.load(base_path+'dxy.npy')

dt = np.load(base_path+'dt.npy')

timestamps = np.load(base_path+'timestamps.npy')

#ranges = np.load(base_path+'ranges.npy')

ranges = np.sqrt(dxy**2 + (dt*zscale)**2)

#event_numbers = np.load(base_path+'event_numbers.npy')

event_select_mask = np.logical_and(ranges > 35, np.logical_and(ranges < 50, counts>6*578472/6.288))

indexes = np.where(event_select_mask == True)

# It looks like the timestamps are in units of 10 ns, so we divide by 1e8 to get seconds

for i in indexes:

timestamps_list.append((f['get']['evt%d_header'%event_num][event_select_mask])/1e8)

timestamps = np.concatenate(timestamps_list)

print(len(timestamps))

hist, bin_edges = np.histogram((timestamps-timestamps[0])/3600, bins=100)

#put everything in units of counts/s

sigma = np.sqrt(hist)/(bin_edges[1] - bin_edges[0])/3600

hist = hist/(bin_edges[1] - bin_edges[0])/3600

#plot

plt.stairs(hist, bin_edges)

bin_centers = (bin_edges[0:-1] + bin_edges[1:])/2

exp = lambda t, A, tau, C: A*np.exp(-t/tau) + C

popt, pcov = opt.curve_fit(exp, bin_centers, hist, p0=(5, 10, 0), sigma=sigma, absolute_sigma=True)

print('half life = %f +/- %f hrs'%(popt[1]*np.log(2), np.sqrt(pcov[1,1])*np.log(2)))

plt.plot(bin_centers, exp(bin_centers, *popt))

print('popt = ',popt)

print('uncertainties: ', np.sqrt(np.diag(pcov)))

plt.yscale('log')

plt.show(block=False)

# plt.savefig('no_flow_half_life.png')

plt.figure()

plt.plot(bin_centers, exp(bin_centers, *popt) - hist)

plt.show()

# plt.savefig('no_flow_half_life_residuals.png')